ELKI 6030 E2 10 – ID 127738

Sound-insulated duct fan with three-dimensionally curved blades

Compact casing made of galvanized steel sheet with very low casing radiation noise. Variable mounting possibilities, in horizontal or vertical position. Direct duct installation via 20mm flange profiles on the casing.

The incorporated diagonal fans make, through their design, a significantly more compact construction size possible and, at the same time, offer a higher flexibility than conventional radial fans. The three-dimensionally curved impeller and stator blades ensure a successful and flow optimized air guidance, which makes the highest aerodynamic efficiency possible. The impeller is balanced according to the quality class G 6.3 of DIN / ISO 1940, in 2 planes. The motor is protected in the hub area and exerts no disturbing influence on the aerodynamics. A built-in thermostatic switch protects the motor against overload. The electrical connection box IP 44 is protected, being installed inside the device casing. The PE-cable gland placed on the casing ensures a simple electrical connection.

The straight flow of the ELKI fans considerably reduces the losses on the downstream installed components. The ELKI series is particularly suitable for noise-sensitive applications in combination with the very efficient duct silencers (type: SDE) which reduce the noise by up to 22 dB(A).

CE marking according to EC Declaration of Conformity for Electromagnetic Compatibility EMC - Directive 2004/108/EC. EC - Declaration of Incorporation according to the Machinery Directive 2006/42/EC – Directive 2009/125/EC (Reg. 1253/2014/EU).

Technical Data:

Volumetric flow (V):

m³/h

Pressure increase (pfa):

Pa

Voltage (U):

1~230 V

Frequency (f):

50 Hz

Power consumption max. (P1):

282 W

Current consumption max. (I):

1,7 A

Rated speed (n):

2890 min-1

Capacitor:

8 µF

Protection class terminal box:

IP44

Inlet sound power level (LWA5):

78 dB(A)

Outlet sound power level (LWA6):

81 dB(A)

Casing sound power level (LWA2):

58 dB(A)

Max. medium temperature (t):

80°C

Dimensions (L x W x H):

357 x 639 x 338 mm

Duct connection:

600 x 300 mm

Weight:

17,9 kg

Wiring diagram:

127818

Manufacturer:

ruck Ventilatoren GmbH

Max-Planck-Str. 5

97944 Boxberg

Tel. 07930-9211300

www.ruck.eu

Type:

ELKI 6030 E2 10

Item number:

127738

Available accessories:

VS 6030		102806		Flexible duct connector size 6030

VKK 6030	103895 		Automatic damper size 6030

SDE 6030 L01	127750		Duct silencer size 6030

GS 01		102787		Isolator switch

TEM 035		103502		5-step transformer with motor protection, max. 3,5 A, (surface mounting)

TES 035		103954 		7-step transformer without protection, max. 3,5 A, (for switch cabinet)